JOHN KARELL, JR., P.E.

121 CUSHMAN ROAD

PATTERSON, NEW YORK, 12563

845-878-7894 FAX 845 878 4939

jack4911@yahoo.com

______________________________________________________________________________

March 13, 2020

RESPONSE TO COMMENTS

Mikes Lawn Service, 351 Wood Street, Putnam Valley (T)

TM # 63.-4-11

BRUCE BARBER, ENVIRONMENTAL CONSULTANT, DATED November 18, 2019

- Project Summary : Noted

- SEQRA: Noted

- Zoning: Need for variance noted.  Application made to ZBA and approved on February 13, 2020.  Copy of the Decision attached.

- Environmental Resources:

Wetlands.  The plans have been revised to include the Ridgebury soils in the Town wetlands boundary and the wetland buffer adjusted.  In addition, the total area of disturbance to the Town wetland and buffer is provided on the plans. A flood plain is not present on this property.

Since all disturbance is outside the 100 foot State Wetland buffer, a DEC permit is not required.  An executed DEC validation block has been added to the plans.  The flagging is valid until October 2020.

A waiver from the tree survey is requested as only a few trees are to be removed.for the septic system construction.

Rain gardens have been provided as opposed to infiltration practices due to the high groundwater table in the front of the property.  The rain garden excavation will be used as a de watering pit during construction, if it is necessary to de water during construction.

A note has been placed on the plans regarding tree cutting due to presence of the long eared bat.

The Office of Parks, etc. has been contacted and they have required an archeological survey of the property.  A copy of the report is attached.

- Review:

- EAF Part 1  -  Many of the below answers were auto filled from the DEC website

Question 12 a.b    DEC auto fill.  Archeological study performed  Report attached.

13b        Added info regarding the area of disturbance within the 100 ft control area.

15          DEC auto fill.  Added timber rattlesnake

16          DEC auto fill

17          Revised

19          Completed.

- Plans

D-1      See item #4 Wetlands above.  Since the area of disturbance is under one acre,

stormwater treatment is not required, the proposed  rain gardens are proposed as

wetland mitigation.

The wetland and wetland 100 foot buffer has been labeled.   An executed  DEC

wetland validation block is provided.

See item  #4above.  A waiver is requested for the tree survey.

I believe that the location of the rain gardens is the most suitable location for the rain

gardens and the NYCDEP has approved the rain gardens in this area as well as the

item 4 driveway.  The flagging of the wetlands should take precedence over the

mapping. As an alternative, Belgian block curbing is proposed  to prevent erosion

into the wetland.

The water supply line from the well to the house has been relocated.

Sight lines are provided.

EC-1     Silt fencing shown

See comment response in item # 4

- The property is located downgradient of the Parkway and will have no impact whatsoever

to the Parkway.  See email from NYSDOT dated January 24, 2020.

Mr. Maiuzzo is the owner of Taconic Wood, Inc.  Deed previously submitted, copy

attached.

TODD ATKINSON, P.E.,   DATED  November 15, 2019

Engineering Review

- Plans are signed and sealed.

- Wetland screening form has been submitted previously, copy attached.  Bruce Barber indicated in his November 18, 2019 memo that he conducted a wetland delineation.

- Survey of the property and the wetlands was performed by Baxter.

- Adjacent property owners are provided on the plans.

- A tree plan waiver is requested.

- Electrical lines are shown.

- All grading is shown on the plans.  Cut and fill amounts are provided.  No significant or quantifiable cut or fill is proposed.  The area of construction of the house and driveway is basically flat.

- The plan includes all proposed drainage facilities.

John Karell, Jr., P.E.